MICROPROCESSOR-BASED SYSTEMS DESIGN

UCS617

Lab Assignment-2 (8086)

Submitted to:

Dr Shivani Sharma

Submitted By:

Dixant Kumar 102103017

Jagdish Agarwal 102103266

Pulkit Arora 102103267

Jeetesh Rajpal 102103268

Janardhan Singh Jadon 102103269

BE Third Year, COE

Group No: 3COE10

Computer Science and Engineering Department

TIET, Patiala

INDEX

| Sr. No. | Title | Page |
| --- | --- | --- |
| 1. | Write an assembly language program to add two 16-bit numbers in 8086. | 1 |
| 2. | Write an assembly language program to subtract two 16-bit numbers in 8086. | 2 |
| 3. | Write an assembly language program to multiply two 16-bit numbers in 8086. | 3 |
| 4. | Write an assembly language program to divide two 16-bit numbers in 8086. | 5 |
| 5. | Write an assembly language program to demonstrate AAA, AAS, AAM, AAD, DAA and DAS in 8086. | 6 |
| 6. | Write an assembly language program to find out the count of positive numbers and negative numbers from a series of signed numbers in 8086. | 13 |
| 7. | Write an assembly language program to find out the largest number from a given unordered array of 8-bit numbers, stored in the locations starting from a known address in 8086. | 15 |
| 8. | Write an assembly language program to find out the largest number from a given unordered array of 16-bit numbers, stored in the locations starting from a known address in 8086. | 17 |
| 9. | Write an assembly language program to print Fibonacci series in 8086. | 19 |
| 10. | Write an assembly language program to perform the division 15/6 using the ASCII codes. Store the ASCII codes of the result in register DX. | 21 |

Program 1

Aim: Write an assembly language program to add two 16-bit numbers in 8086.

Sol:

MOV AX,1234H

MOV BX,1236H

ADD AX,BX

HLT

Output:

Fig 1.1

Fig 1.2

Program 2

Aim: Write an assembly language program to subtract two 16-bit numbers in 8086.

Sol:

MOV AX, 1234H

MOV BX, 1236H

SUB AX, BX

HLT

Output:

Fig 2.1

Fig 2.2

Program 3

Aim: Write an assembly language program to multiply two 16-bit numbers in 8086.

Sol:

MOV AX, [0301H]

MOV BX, [0303H]

MUL BX

HLT

Input:

Fig 3.1

Fig 3.2

Output:

Fig 3.3

Fig 3.4

Program 4

Aim: Write an assembly language program to divide two 16-bit numbers in 8086.

Sol:

MOV AX,5600H

MOV BX,2500H

DIV BX

HLT

Output:

Fig 4.1

Fig 4.2

# Program 5

Aim: Write an assembly language program to demonstrate AAA, AAS, AAM, AAD, DAA and     DAS in 8086

# Sol:

| AAA | AAS |
| --- | --- |
| MOV AX,0032H | MOV AL,0033H |
| MOV BX,0033H | SUB AX,0039H |
| ADD AX,BX | AAS |
| AAA | OR AL,0030H |
| HLT | HLT |

| AAM | AAD |
| --- | --- |
| MOV AL,03H | MOV AX,0033H |
| MOV BL,09H | MOV BX,0032H |
| MUL BL | AAD |
| AAM | DIV BX |
| OR AX,3030H | HLT |
| HLT |  |

| DAA | DAS |
| --- | --- |
| MOV AL,71H ADD AL,43H DAA HLT | MOV AL,71H SUB AL,43H DAS HLT |

AAA:

Fig 5.1

Fig 5.2

AAS:

Fig 5.3

Fig 5.4

AAM:

Fig 5.5

Fig 5.6

AAD:

Fig 5.7

Fig 5.8

DAA:

Fig 5.9

Fig 5.10

DAS:

Fig 5.11

Fig 5.12

Program 6

Aim: Write an assembly language program to find out the count of positive numbers and negative numbers from a series of signed numbers in 8086.

Sol:

MOV CL,0AH

MOV BL,00H

MOV DL,00H

LEA SI, [1000H]

L1: MOV AL, [SI]

SHL AL, 01

JNC L2

INC DL

JMP L3

L2: INC BL

L3: INC SI

DEC CL

JNZ L1

MOV [100AH], BL

MOV [100BH], DL

HLT

Input:

Fig 6.1

Output:

Fig 6.2

Fig 6.3

Program 7

Aim: Write an assembly language program to convert to find out the largest number from a given unordered array of 8-bit numbers, stored in the locations starting from a known address in 8086.

Sol:

MOV CL, 0AH

LEA SI, [1000H]

MOV AL, [SI]

L1: INC SI

MOV BL, [SI]

CMP AL, BL

JC L2

JMP L3

L2: MOV AL, BL

L3: DEC CL

JNZ L1

MOV [100AH], AL

HLT

Input:

Fig 7.1

Output:

Fig 7.2

Fig 7.3

Program 8

Aim: Write an assembly language program to find out the largest number from a given unordered array of 16-bit numbers, stored in the locations starting from a known address in 8086.

Sol:

MOV BX, 1000H

MOV C L, [BX]

INC BX

MOV AX, [BX]

DEC CL

Back: INC BX

INC BX

CMP AX, [BX]

JNC Next

MOV AX, [BX]

Next: DEC CL

JNZ Back

MOV [1020H], AX

HLT

Input:

Fig 8.1

Output:

Fig 8.2

Fig 8.3

Program 9

Aim: Write an assembly language program to print Fibonacci series in 8086.

Sol:

MOV AL,00H

MOV SI,500H

MOV [SI], AL

ADD SI,01H

ADD AL,01H

MOV [SI], AL

MOV CX, [0000H]

SUB CX,0002H

L1:MOV AL, [SI-1] 
ADD AL, [SI]

ADD SI,01H

MOV [SI], AL

LOOP L1

HLT

Input:

Fig 9.1

Output:

Fig 9.2

Fig 9.3

Program 10

Aim: Write an assembly language program to perform the division 15/6 using the ASCII codes. Store the ASCII codes of the result in register DX.

Sol:

MOV AX, ‟15‟

MOV BX, “6‟

SUB AX, 3030H

SUB BH, 30H

AAD

DIV BH

ADD AX, 3030H

MOV [SI], AX

HLT

Output:

Fig 10.1

Fig 10.2